VySkan

| Product Name | 4G LTE Advanced Telematics Terminal |
| --- | --- |
| Product Model | VF50R |

Versatility. Precision. Seamless Integration.

Engineered for complex fleet management and heavy-duty logistics, the VF50R is a highly expandable 4G LTE tracking terminal. It bridges the gap between precise GNSS tracking and deep vehicle telemetry by offering rich I/O interfaces, CAN bus data reading, and hassle-free software integration.

- Core Value

- Ultimate I/O Expandability

Beyond simple tracking, the VF50R is equipped with RS232/RS485 and CAN bus interfaces. Easily integrate with external cameras, RFID readers, and extract rich OBD vehicle telemetry to build a customized, comprehensive IoT solution.

- Zero Data Loss Guarantee

Never lose a track. Built-in dynamic storage automatically logs telemetry data during cellular signal blind spots and seamlessly uploads it upon reconnection, ensuring 100% data integrity for your fleet's routes.

- Intelligent Power Management

Featuring a wide 9V-50V operating range and an ultra-low sleep current (5-15mA), it protects vehicle batteries while providing active defense through power-loss and geo-fencing alerts.

- Target Applications

- Heavy-Duty Logistics & Freight (CAN bus/OBD tracking)

- Enterprise Fleet Management (Vehicle status monitoring)

- Public Transport & Passenger Vehicles (Camera & RFID integration)

- Construction & Industrial Equipment (Asset utilization)

- Technical Specifications

3.1 Cellular & GNSS

| Category | Specification | Description |
| --- | --- | --- |
| Cellular Module | Quectel EC20 | Industrial-grade 4G LTE communication module. |
| Network Bands | 4G LTE-FDD/TDD 3G WCDMA / 2G GSM | Broad coverage for global fleet operations with seamless fallback. |
| SIM Card | Standard SIM | Accessible slot for easy deployment. |
| GNSS Module | ZKMicro AT6558D (GPS+BDS) | High-performance dual-satellite tracking chip. |
| Positioning Accuracy | 5 - 10 meters | Pinpoint precision for real-time asset visibility. |
| Start Time | Cold: Avg 32s Hot: Avg 1s | Rapid satellite acquisition supported by AGPS. |

3.2 Power & Electrical

| Category | Specification | Description |
| --- | --- | --- |
| Operating Voltage | DC 9V - 50V | Wide voltage range compatible with light and heavy-duty vehicles. |
| Working Current | 30 - 60 mA @ 12V | Standard active tracking mode. |
| Sleep Current | 5 - 15 mA @ 12V | Ultra-low power consumption mode. |
| Backup Battery | 180 mAh / 3.7V (Optional) | Triggers main power-loss alarms for active defense. |

3.3 Interfaces & Expandability

| Category | Specification | Description |
| --- | --- | --- |
| Digital Inputs | 2x High (ACC/Doors) 1x Low (SOS/Seatbelt) | Comprehensive monitoring of vehicle status. |
| Digital Outputs | 1x Low Output | Remote fuel/power cut-off or door lock control. |
| Serial Ports | 1x RS232 / RS485 | Integrates with external cameras and RFID readers. |
| Vehicle Bus | 1x CAN Bus | Reads deep automotive OBD diagnostic data. |
| Audio | 1x Mic Input | Supports parallel external microphones for voice monitoring. |

3.4 Physical & Environmental

| Category | Specification | Description |
| --- | --- | --- |
| Dimensions | 87mm × 64mm × 26mm | Compact footprint for covert installation. |
| Weight | 140g | Robust yet lightweight design. |
| Housing Material | ABS Plastic | Industrial-grade durability. |
| Operating Temp. | -30℃ to 75℃ | Reliable operation in extreme climates. |
| Mounting | Magnetic Support | Optional strong magnetic mount for versatile deployment. |

- Built for Rapid Integration & Telemetry

We know hardware is only half the solution. The VF50R is designed to drastically reduce your Time-to-Market (TTM) and development costs.

- Standard Protocols

Natively supports TCP/UDP and the industry-standard GT06 protocol.

- Supported Platforms

Seamlessly integrates with industry-leading tracking software including Wialon, Traccar, and enterprise custom dashboards without reverse engineering.

- CAN Bus Ready

Comprehensive Open API documentation helps your IT team effortlessly parse not just GPS coordinates, but deep OBD telemetry data, achieving live dashboard connectivity in under 48 hours.

- A personalized service prepared for you

- 1×VF50R Sample Device (Pre-configured)

- Full API & Protocol Documentation

- 1-on-1 Technical Support from our Field Engineers

- Contact us

Website：https://vyskan.com/

Email: marketing@vyskan.com