VySkan

| Product Name | 2G Wide-Voltage GPS Vehicle Tracker |
| --- | --- |
| Product Model | VM90-2G |

Reliable. Cost-Effective. Wide-Voltage.

Engineered for cost-sensitive fleet management and auto finance operations, the VM90-2G is a highly stable 2G vehicle tracker featuring GSM wireless communication and GPS/BDS satellite navigation. Featuring a built-in industrial-grade antenna and a highly integrated design, it delivers essential real-time tracking, asset protection, and intelligent power management in a highly cost-effective package.

- Core Value

- Universal 9V-90V Compatibility

Designed to operate seamlessly across an ultra-wide DC 9V to 90V range, making it a universal solution for everything from e-bikes and passenger cars to heavy-duty trucks.

- Intelligent Power Management

Built with a 3-axis accelerometer and a smart sleep/wake mechanism, the device automatically enters sleep mode (drawing less than 10mA) when stationary to protect the vehicle's battery life.

- Comprehensive Remote Asset Defense

Supports multiple security alerts, including vibration, power-cut, over-speed, illegal start, and displacement alarms. It also allows fleet managers to execute remote fuel and power cut-off commands.

- Zero Data Loss Architecture

Equipped with a built-in large-capacity storage chip, it automatically caches trajectory data when entering cellular blind zones and seamlessly uploads the stored data once the network is restored.

- Target Applications

- Insurance & Auto Finance: (Risk management, illegal start alerts).

- Corporate Fleets & Logistics: (Wide voltage, remote fuel/power cut).

- E-bikes & New Energy Vehicles: (9-90V support, low power consumption).

- Technical Specifications

  - Cellular & GNSS

| Category | Specification | Description |
| --- | --- | --- |
| Network Bands | 2G GSM:850/900/1800/1900MHz | Global 2G quad-band support. |
| SIM Card | Micro SIM | Built-in push-push slot. |
| GNSS Module | GPS + BDS | High-sensitivity dual-star positioning. |
| Positioning Accuracy | < 10 meters | Reliable real-time asset visibility. |
| Start Time | Cold: Avg 32s ¦Hot: Avg 1s | Rapid satellite acquisition supported by AGPS. |

  - Power & Electrical

| Category | Specification | Description |
| --- | --- | --- |
| Operating Voltage | DC 9V - 90V | Ultra-wide range compatible with a vast array of vehicles. |
| Working Current | Avg 35mA @ 12V | Standard active tracking mode. |
| Sleep Current | < 10mA @ 12V | Ultra-low power consumption. |
| Backup Battery | 80 mAh / 3.7V Li-Polymer (Optional) | Enables power-cut alarms upon main power disconnection. |

3.3  Interfaces & Expandability

| Category | Specification | Description |
| --- | --- | --- |
| Smart Alarms | Vibration, Power-cut, Over-speed, Illegal Start | Multi-layered alert system for proactive security. |
| Vehicle Control | 1 Channel Fuel/Power Cut | Supports external relay for remote immobilization. |
| Ignition Detection | 1 Channel ACC Detection | Accurately monitors vehicle start/stop status. |

3.4 Physical & Environmental

| Category | Specification | Description |
| --- | --- | --- |
| Dimensions | 75mm × 27mm × 14mm | Compact footprint. |
| Weight | 30g | Lightweight design. |
| Housing Material | ABS Plastic | Industrial-grade durability. |
| Operating Temp. | -20℃ to 75℃ | Reliable operation in diverse conditions. |

- Built for Rapid Integration

- Standard Protocols: Natively supports TCP transmission and offers a choice between standard JTT808 or GT06 protocols.

- OTA Ready: Supports dual-mode remote upgrades (Platform + SMS) to minimize maintenance costs.

- A personalized service prepared for you

- 1× VM90-2G Sample Device (Pre-configured)

- Full API & Protocol Documentation

- 1-on-1 Technical Support from our Field Engineers

- Contact us

Website：https://vyskan.com/

Email: marketing@vyskan.com