VySkan

| Product Name | 4G LTE Intelligent OBD Vehicle Tracker |
| --- | --- |
| Product Model | OBD Standard |

Compact. Intelligent. Reliable.

Engineered specifically for auto finance risk management and fleet monitoring, this intelligent OBD terminal delivers uncompromising security and real-time visibility. Its ultra-compact design makes it the smallest in its performance class, perfectly suited for covert installations and seamless plug-and-play deployments.

- Core Value

- Ultra-Compact & Stealth Deployment

With dimensions of just 51mm × 47mm × 23mm, it is designed for highly concealed installations within confined vehicle spaces.

- Intelligent Power Management

A built-in 3D 3-axis accelerometer enables a smart sleep/wake mechanism. When stationary, it drops power consumption to an ultra-low 2.5mA, actively protecting the vehicle's battery while maintaining defense.

- Uninterrupted Trajectory & Data Security

Network blind spots are mitigated by a built-in cache that stores up to 800 data points when cellular connection drops. Combined with cornering compensation logic, it ensures 100% precise route mapping.

- Proactive Asset Defense

Equipped with comprehensive multi-alert systems including low-voltage alarms (triggered after 5 minutes of external power disconnection) , over-speed, and under-speed alerts to ensure maximum fleet safety.

- Target Applications

- Auto Finance & UBI Insurance (Risk management, minimal footprint)

- Corporate Fleet Management (Mileage, speed monitoring, and precise trajectory)

- Car Rental & Leasing (Plug-and-play installation & asset security)

- Technical Specifications

  - Cellular & GNSS

| Category | Specification | Description |
| --- | --- | --- |
| Network Bands | 2G/4G/4G+2G (GSM,FDD-LTE, TDD-LTE) (Global/Regional bands optional ) | Flexible network configuration based on client requirements. |
| GNSS Module | GPS/BDS/ GLONASS | Multi-constellation support for high-performance satellite tracking. |
| LBS Fallback | Supported | Automatically switches to base station tracking (30s intervals) in GPS blind zones. |
| Sensitivity | Tracking:-162 dBm | Maintains strong signal lock. |
| Start Time | Cold: Avg 32s ¦ Hot: Avg 1s | Rapid satellite acquisition. |

  - Power & Electrical

| Category | Specification | Description |
| --- | --- | --- |
| Operating Voltage | DC 9V - 35V | Wide voltage range compatible with most vehicles. |
| Working Current | Avg 35mA (ACC ON) | Standard active tracking mode. |
| Sleep Current | 2.5mA (ACC OFF) | Ultra-low power consumption mode. |

  - Interfaces & Expandability

| Category | Specification | Description |
| --- | --- | --- |
| Smart Alarms | Low-voltage, Over-speed, Under-speed | Comprehensive built-in alerts for fleet safety and security. |
| Smart Sensor | Built-in 3D 3-axis Accelerometer | Detects vehicle motion and manages intelligent sleep logic to protect battery life. |
| Data Cache | Blind Zone Compensation (Up to 800 logs) | Automatically caches trajectory data during GPRS dropouts and uploads upon reconnection. |
| Protocols | JTT808 / GT06 | Natively supports industry-standard protocols for seamless platform integration. |

  - Physical & Environmental

| Category | Specification | Description |
| --- | --- | --- |
| Dimensions | 51mm × 47mm × 23mm | Smallest in its class for covert installation. |
| Weight | 53g | Extremely lightweight design. |
| Housing Material | ABS Plastic | Industrial-grade durability. |
| Operating Temp. | -25℃ to 75℃ | Reliable operation in varied climates. |

- Built for Rapid Integration

- Standard Protocols: Natively supports TCP transmission and offers a choice between standard JTT808 or GT06 protocols.

- OTA Ready: Supports dual-mode remote upgrades (Platform + SMS) to minimize maintenance costs.

- A personalized service prepared for you

- 1× OBD Standard Sample Device (Pre-configured)

- Full API & Protocol Documentation

- 1-on-1 Technical Support from our Field Engineers

- Contact us

Website：https://vyskan.com/

Email: marketing@vyskan.com